DRAFT – February 20, 2019

Site Plan Requirements for New Builds

Existing Conditions Site Plan

- Provide a scaled site plan showing as applicable: footprint of existing house, garage and other structures; hardscape elements such as the driveway, patio(s) and walkway(s); fencing; property lines; easements; setback lines; Right-of-Way, etc.  Include bar scale and north arrow.

- Location of existing trees 6” caliper and greater.  Note caliper, botanical and common names, and condition.  Calculate and note total caliper of existing trees on site.

- Location of existing ornamental trees and major shrubs (36” tall or greater).  Note size, botanical and common names.

- Location of street trees in Right-of-Way.  Identify trees to be protected from construction on both private property and within Right-of-Way.  Provide a tree protection plan approved by City staff.

- Location of utility poles.

- Identify areas of natural screening and major areas of shrubs along the perimeter of the lot.

- Identify trees to be removed and total number caliper inches to be removed.  Identify major shrubs to be removed.

- Identify elevation changes on property, if significant.

- Identify if street has brick curb and gutters.

- Provide several site photos to illustrate existing conditions and adjacent residences.

Proposed Construction Site Plan

- Provide a scaled site plan showing as applicable: footprint of existing house, garage and other structures; hardscape elements such as the driveway, patio(s) and walkway(s); fencing; property lines; easements; setback lines; Right-of-Way, etc.  Include bar scale and north arrow.

- Location of proposed new landscape – trees, shrubs, and perennials.  Note quantity, size, botanical and common names.  Calculate and note total caliper of proposed trees on site.

- Location of street trees in Right-of-Way.

- Location of utility poles.

- If existing natural screening along perimeter of the lot is removed, identify replacement landscape.

- Identify elevation changes on property, if significant.

- Identify if landscape will be irrigated.

- If property has brick curb and gutters, provide detail for protecting and/or replacing brick when construction is completed.

- Provide a brief narrative describing the proposed landscape plan, including the relationship of new construction and landscaping to neighboring residences.